The high absorbing, interacting and reacting capabilities of NP’s make it useful for water treatment in depths and any location generally left out by other conventional technologies. There are various recent advances on different nanomaterials (nanostructured catalytic membranes, nanosorbents, nanocatalysts, bioactive nanoparticles, biomimetic membrane and molecularly imprinted polymers (MIPs)) for removing toxic metal ions, disease causing microbes, organic and inorganic solutes from water.

Nanosorbents

Nanosorbents have very high and specific sorption capacity having wide application in water purification,

remediation and treatment process.

Magnetic nanosorbents also helps in treating waste water and is proved very interesting tool especially for

organic contaminants removal (Bull Mater, 2011)8. Since most of the contaminants are not of magnetic

nature filtration aids are needed to absorb which is generally followed by magnetic separation. The

nanosorbents used for magnetic separation are prepared by coating magnetic nanoparticles with specific

ligands presenting specific affinity (Apblett et al., 2001)9. Different methods like magnetic forces, cleaning

agents, ion exchangers and many more are used to remove nanosorbents from the site of treatment to avoid

unnecessary toxicity. Regenerated nanosorbents are always cost effective and promoted more for

commercialization.

Adsorption: Use of nanomaterials in treatment of waste water

is applicable in various forms like catalytic, absorptive, catalytic

membrane, bioactive nanoparticles, biomemetic membrane,

polymeric and nanocomposite membrane, thin film composite

etc. Various organic chemicals10 are absorbed more efficiently

by using carbon nano tubes (CNT) than activated carbon

.Organic compounds which have carboxylic, hydroxyl, amide

functional groups also form hydrogen bond with the graphitic

CNT surface which donates electrons11.CNT have high

adsorption competence for metal ions12-14 and therefore are a

good alternative for activated carbon.

Nanoscale metal oxides like iron oxides like ferrous oxide,

TiO2, Al2O3 are effective ,low cost adsorbants for heavy metals

And radio nucleides15-17. Dendrimers [polymeric nanomaterials]

are capable of removing both organics and heavy metals18.

Nanoadsorbents are used as powder, beads or porous granules

loaded with nanoadsorbants.

Nanomembranes: Membranes with nanofibres can remove

micro-size particles from aqueous phase with a high elimination

rate without considerable fouling19. Such membranes are used

as pretreatment method used proceeding to ultrafiltration or

reverse osmosis. Large number of studies on membrane

nanotechnology has focused on creating multifunction

membrane by adding nanomaterials into polymeric or inorganic

membranes known as nanocomposite membranes. The addition

of metal oxide nanoparticles including alumina20, silica21,

zeolite22 and TiO2

23 to polymeric ultra filtration membranes has been shown to amplified membrane surface hydrophilicity,

water permeability, or fouling resistance. Antimicrobial

nanomaterials such as nanosilver is doped or surface grafted on

polymeric membranes to inhibit bacterial attachment and biofilm

formation24,25 on the membrane surface It also as

inactivates viruses26 and can reduce membrane bio-fouling

Development of Thin film Nanomaterial membrane mainly

focuses on incorporating nanomaterials into the active layer of

thin film composite [TFC] membranes via doping in the casting

solutions or surface modification. The effect of nanoparticles on

Membrane permeability and selectivity depends on the sort,

dimension and quantity of nanoparticles added. Many biological

inspired membranes are highly selective and permeable27,28

Nanocatalysts: Due to high surface to volume ratio and shape

dependent properties nanocatalytic substances like zero-valent

metal, semiconductor materials and bimetallic nanoparticles are

widely used in water treatment as they increase the catalytic

activity at the surface. It enhances the reactivity and degradation

of environmental contaminants such as organochlorine

pesticides, halogenated herbicides azo dyes, polychlorinated

biphenyls and nitro aromatics29. The catalytic activity has been

proved on laboratory scale for various contaminants. Reports

are available which show that efficient for degradation of

microbial contaminants in water and reusable particles30 are

silver nanocatalyst, N-doped TiO2 and ZrO2 nanoparticles.

Nanostructured catalytic membranes: Nanostructured

catalytic membranes have more than a few advantages like they

have homogeneous catalytic sites, require less contact time

,multiple ordered reactions can occur and they can be easily

scaled for commercial purpose .To inactivate microorganisms

and decompose organic pollutants nanostructured TiO2

membranes and films are used31. With the advancement in

nanotechnology several novel nanostructured catalytic

membranes has been synthesized. These have with increased

foul resistance, higher selectivity and higher rate of

decomposition. The techniques of synthesizing these

nanoparticles include many approaches for developing their

multi functionality32.

Bioactive nanoparticles: Silver nanoparticles can be

biosynthesized extracellularly by bacteria Bacillus cereus which

is having very high antibacterial potential. This strain was

exposed to different concentrations of silver salt [AgNO3] and

studied with the help of various analytical instruments like High

Resolution Transmission Electron Micrography [HRTEM], Xray

diffraction [XRD] and Energy Dispersive spectroscopy

[EDS] Prakash et al.33 found MgO nanoparticles and Cellulose

acetate [CA] fibers embedded Ag nanoparticles effective against

bacteria both positive and negative and spores as well. With the

rapid increase in nanotechnology approaches for the detection of

microbial pathogens is continuingly adding to the microbial and

pathogen detection as well as in diagnostics.

Biomimetic membrane: Biomimetic membranes are chemically stable, have high permeability and selectivity34 and

show a great degree of removing salts.

A brief summary is shown in table-1 describing application of

nanotechnology in water treatment